Delete highlighted text before printing.

SECTION 11 8226 – WASTE COMPACTORS

~ WASTE COMPACTORS CONTAINER PACKER

PART 1 – GENERAL

  - RELATED DOCUMENTS

- The general provisions of the contract, including General and Supplementary Conditions and Division 1, General Requirements, apply to the work specified in this section.

- Related Sections:

  - Section 03 0110  – Concrete Forms and Accessories: placement of anchors, bolts and inserts - by others.

  - Section 14 9182 – Chutes: Rubbish/Trash Chute.

  - Division 26 0000:  Electrical supply-system connections - by others.

- Section Includes:

  - Container Waste Compactor and Compaction Containers.

- Description of Work

  - Work Included:  Furnish a waste compactor where shown on drawings.

  - SUBMITTALS

- See Section 01 7700 – Administrative Requirements, for submittal procedures.  General Contractor to furnish Subcontractor approved shop drawings and plan view drawing of trash room.

  - Product Data:  Provide unit capacities, physical dimensions, utility requirements and layout.

  - Shop Drawings:  Indicate machine location and clearances required.  Permanent disconnect located within 4’ of trash chute discharge.

- Close-out Submittals:

  - Operation and Maintenance Manual (O&M Manual).

  - Warranty Documents: Issued and executed by the manufacturer and installer of the system.

  - QUALITY ASSURANCE

Recommended qualifications listed below.  Delete/Edit subparagraphs as needed.

- Qualifications:

  - Manufacturer: Minimum five (5) years documented experience-producing products specified in this section.

  - Installer: Approved by the Manufacturer, and having a minimum of five (5) years experience.

  - Products must be manufactured in the United States.

- General Contractor shall coordinate all electrical requirements and characteristics of building electrical system with Owner in order to provide for efficient operation of equipment.  General Contractor shall ensure that all required electrical conduits, dedicated circuit breakers and disconnects are installed and ready for use for electrical load required by equipment and operation, prior to equipment being delivered to site.

  - WARRANTY

- Manufacturer’s warranty: Furnish manufacturer’s standard one (1) year warranty from date of installation.  Warranty shall apply to defects in product workmanship and material only.

PART 2 – PRODUCTS

2.1	MANUFACTURER

- Basis of Design:

  - CHUTES International Manufacturing, 33 Industrial Park Dr., Waldorf, Maryland  20602; Telephone: (800) 882-4883; FAX (301) 753-4109. www.chutes.com or info@chutes.com

  - Substitutions:  See Section 01 6000- Product Requirements.

  - WASTE COMPACTORS

- General Contractor shall provide Subcontractor with approved shop drawings indicating all operational information, including but not limited to, electrical power requirements.

In subparagraph below, select desired compactor

- [DuraPak Model DP130-HD horizontal compactor with chute fed hopper.  Required minimum room size for this standard machine, compaction can connected and extra compaction can in room is 14’ x 12½’ with proper chute location.]

[DuraPak Model DP130-HDS Short-Body horizontal compactor with chute fed hopper.  Required minimum room size for this standard machine, compaction can connected and extra compaction can in room is 12’ x 12½’ with proper chute location.]

- Capacity per hour:  20 cubic yards with 45-second cycle time.

- 5 HP electrical motor with ‘UL’ listed control panel

In subparagraphs below, select power requirement

- Electrical:  [208V/230V] [460V/480V], 3-phase, 5 wire, 60 hertz, 30 amp.

- 14-Gallon hydraulic oil tank with 4 GPM pump.

- Programmable Logic Control (PLC).

- Low friction, noise reducing wear pads.

- Oil pressure gauge.

- On/Off key switch, mushroom stop button, ‘On’ light, ‘Container Full’ light, ‘Excessive Run’ indicator lamp, photo electric eye for automatic cycling, ‘Overload’ indicator lamp, night-shut down, and ram retraction button.

- Access door with safety interlock switch.

- Single solenoid soft shift directional valve.

- Hinged steel wiper blade on ram.

- Discharge containers OPTIONAL ADD

In subparagraphs below, select desired ADD OPTIONS; delete as needed

  - Compaction can [2-cubic yard front load] or [2-cubic yard rear load] [3 cubic yard front load only] reinforced and specifically designed for internal compaction.

  - Non-Compaction can [2-cubic yard front load] or [2-cubic yard rear load] [3 cubic yard front load only] reinforced container can.

  - 2 Fixed, 2 Swivel 6” Casters

PART 3 – EXECUTION

  - EXAMINATION

- Verification that anchors are correctly secured.

- Permanent disconnect to be located within 4’ of trash chute discharge, by others.

  - INSTALLATION

- Install unit in accordance with shop drawings, manufacturer’s installation instructions and authority having jurisdiction.

- Coordinate with trash chute discharge.

  - Anchor unit securely in place.

  - Electrical:  rough-in and final connections under Division 26.

  - Hydraulic system:  take precautions to keep dust, dirt, moisture and other foreign matter out of hydraulic system

  - Start Up:  comply with manufacturer’s O&M manual.

- Use for construction trash:  NOT ALLOWED.

  - DEMONSTRATION

- Demonstrate and instruct Owner on unit operation.  ALL associated users of compactor must be present at demonstration.

END OF SECTION 11 8226 –WASTE COMPACTORS – CONTAINER PACKER